| 1 | Delivering, unloading and storing of SMDB & DB materials using pickup and unloading by manual handling | Plant and Traffic movement - Run over by vehicles/plants - Hit by reversing equipment - Overhead obstructions - Vehicle break down - Property damage - Collision with other vehicles - Personal injury - Fatality Accident due to poor lighting and visibility (operation at night | Operatives Staff/visitors | 3 | 4 | 12 | Medium | 1) Delivery/ collection drivers shall be subject to McLaren induction/Training with regards to traffic management. 2)Ensure exclusion zone is set up around the preparation work area and all plant movement and checked by supervisor 3) Always use the pedestrian access provided 4) Traffic light batons shall be provided for banks man at night 5) All plant to have a fully trained banks man present all times. 6) No plant to reverse without banks man. 7) Ensure access route are sufficient and safe to use. 8) Do not take rest in or under vehicles 9) Proper barricade and safety signboard provided on open excavation 10) All vehicles must be fitted with reverse alarm / Flashing light. 11) The work place and all access to be well illuminated. 12) Ensure close supervision, Effective Communication & Coordination with McLaren 13) Enforce Speed limit 14) Ensure all vehicles entering the site is properly maintained and reported if found any defect 15) Ensure materials are secured/tied in pallet trolley to avoid fall from trolley. | 1 | 4 | 4 | Low | Site Eng. Supervisor, Foreman, |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Manual Handling | musculoskeletal disorders, Hand injuries, cut , bruises Sharp edges | Operatives involved in work | 3 | 4 | 12 | Medium | 1) Eliminate the need for manual handling by using mechanical aid. 2)Reduce the weight of a load to limit force exertion 3) If unsure of the load weight, check with supervisor. 5)Ensure proper manual handling procedure 6) Identify and eliminate sharp edges before any manual lift 7) Ensure adherence with sufficient and appropriate PPE. 8) Limit load carrying to 20kg per person 9) Supervisor must make sure while manual handling load individual task and environment to be considered to reduce the risk of manual handling. | 1 | 4 | 4 | Low | Supervisor, Foreman, Site Eng. |
| 3 | Material storing | Slips, Trips and fall Poor house keeping | Operatives Staff/storekeeper | 3 | 4 | 12 | Medium | - Ensure accesses are kept clear at all times. 2)Housekeeping to be carried out regularly throughout the day when required 3)Supervisor to ensure operatives are stacking material neatly and stacks are stable and in a safe condition 4)Provide adequate lighting especially during night hours 5) Appropriate space is allowed around storage areas for employees to move around safely without the risk being trapped between stacked materials. 6) Maintain good housekeeping. | 1 | 4 | 4 | Low | Supervisor, Foreman, Site Eng. |
| 4 | SMDB & DB fixing by manual handling | musculoskeletal disorders, Hand injuries, cut , bruises Sharp edges Tripping hazard Fall from hand by human error | Involved all operatives | 3 | 4 | 12 | Medium | 1 Eliminate the need for manual handling by using mechanical aid. 2)Reduce the weight of a load to limit force exertion 3) If unsure of the load weight, check with supervisor. 5)Ensure proper manual handling procedure 6) Identify and eliminate sharp edges before any manual lift 7) Ensure adherence with sufficient and appropriate PPE. 8) Limit load carrying to 20kg per person 9) Supervisor must make sure while manual handling load individual task and environment to be considered to reduce the risk of manual handling. 10) Ensure accesses are kept clear at all times. 11) Ensure close supervision and safe manual handling with full attention on work. | 1 | 4 | 4 | Low | Site Eng. Supervisor Foreman |
|  | Installation of SMDB/DB using Mobile tower | Work at height Fall form height Falling objects Over loading Access & egress | Operatives/Staff/ Visitors Others | 3 | 4 | 12 | Medium | 1) Only trained and authorised personnel to carry out the task. 2) Ensure the PTW in place. 3) Ensure proper working platform with complete fall protection 4) Ensure Full body harness and 100% Tie off 5) Ensure the scaffold platform is properly erected with "Safe to use" tag. 6) Ensure castor wheels are locked and out rigger in place. 7) Hand tools must be secured/tethered 8)Work materials strictly not allowed to be stored on platform except for working tools 9) Always comply with the WLL & man loading capacity 10) Ensure unauthorised person not allowed to modify the scaffolding and if needed to any changes then must be modified by trained and certified scaffolder. 11)Ensure safe access to the working platform 12)Scaffold access must be free from obstruction 13) Work area must barricade with signage and watchman must monitor no unauthorised person will enter inside exclusion zone. 14) Ensure sufficient illumination in work area. 15) Work area should clean and remove all the debris end of shift. 16) Ensure that working near fragile surface special care of body and eye protection with necessary PPE. | 1 | 4 | 4 | Low | Supervisors Foreman, Site Eng. |
| 6 | Use of Ladder During installation of SMDB & DB | Falling from height Working on Uneven ground Falling objects | Operatives/ Staff/ | 3 | 4 | 12 | Medium | 1) Only light work, off short duration and work in which the operative can maintain secure hand and foot hold can be undertaken form a ladder 2) Never try to overreach with any ladders 3) Check the ladder is of suitable quality for industrial use and is in good condition 4) Maintain a minimum of 3 points of contact with stepladders at all times (feet/thighs/hands) If possible avoid the use of stepladders at a working height of 2 meters and more. 5) Check the ladder legs (and stays) are fully deployed or locked (depending on type) to maintain maximum base dimensions and the step ladder is orientated to provide maximum stability. 6) Stepladders/ladders has to be used on leveled ground/firm base 7) Ensure that during using ladder from the top 2 step is blocked to avoid climb on that. 8) The ladder securely fixed to prevent slipping outwards or sideways or securely footed at all times. 9) Person must hold the ladder while another operative working on it 8) Materials should not be placed above the thread of ladders/step ladders 9) Do not carry any materials in one hand while climbing or getting down from ladder 10) Ensure the area under the ladder is barricaded especially if it is being used in a public area. 11) Ensure the ladder is electrical insulated podium type if working in live services | 1 | 4 | 4 | Low | Supervisor Site Eng. Foreman, |
| 7 | Cable Glanding & termination using Hole saw cutter | Entanglement Noise Laceration Sharp edge | Involve operatives | 4 | 3 | 12 | Medium | 1.Hot work permit should be obtained and precaution for hot work 2. Equipment/power tools should be inspected Before use. 3.Equipment/power tools should be used by Trained & experienced workers. 4 Ensure skill person in work and avoid entanglement with body parts/finger and dress 5. Use Industrial socket for plugging. 6.Do not make any changes of the safety device of the power tools 7. use noise protection and eyed protection 8.Powertools should be colour coded and 110 volt 9. Never hang/lift the machine by using its power cable cord 10. Operator shall ensure the drilling/grinding machine is fitted low/high speed. 11. Use proper gloves to protect from sharp edge tools. 12. Fire extinguisher must be in place 13. Ensure good housekeeping. 14. Avoid using cutting pliers to turn screws or bolts. Never use them in place of a wrench/hammer or screwdriver 15. Ensure to protect your eyes from any particles of splinters, always wear good quality safety glasses when you work with crimping pliers. | 2 | 3 | 6 | Low | Supervisor, Foreman, Safety officer |
| 8 | Use of power tools such as angel grinder, Drill machine, | Damaged cable insulation, Sockets. Electrocution Electric shock Short circuit Electric burn Trip/Fall Noise Hearing loss Vibration Dust | Operatives and staff | 3 | 4 | 12 | Medium | 1) All portable electrical equipment must be PAT tested and color coded. Make sure the test dates are visible on the equipment. 2) Ensure all power tools are 110 volt and peridic inspection by qualified electrician. 3) PTW to be obtained, prior to commencing the task 4) Damaged industrial sockets and power cables must be removed. 5) Ensure the electrical cable is not damaged and has not been repaired with insulating tape or unsuitable connectors. 6) Ensure cables from power tools shall be organized so as not to present a tripping hazard 7) Only trained competent operatives to operate the power tools. 8) Ensure proper and regular maintenance of equipment that takes account of noise 9) Ensure proper hearing protection always. 10) Make sure people use the right tool for the job and are trained to use it correctly 11) Ensure defective tools that require maintenance is reported for repair or replacement. 12) Instruct workers to keep their hands warm and dry, and to not grip a vibrating tool too tightly. Workers should allow the tool or machine to do the work. 13) Ensuring proper job rotation and limiting time operatives working with power tools and hand tools. 14) Ensure adequate earth leakage protections are provided. 15) Never hang/lift the machine by using its power cable cord 16) Operator shall ensure the drilling/grinding machine is fitted low/high speed. | 1 | 4 | 4 | Low | Supervisor Foreman Site Eng. |
| 9 | Working under High temperature Direct sun light Dusty High wind | Heat exhaustion, Heat stroke, Dehydration, Personal injuries due to high temperature, Burn, Tiredness | Workers | 4 | 4 | 16 | High | 1.Arrange adequate drinking water and rest shelter 2.Drink plenty of water with electrolyte 3.If feel something unusual immediately inform supervisor and report to site clinic. 4.Make shift to operate the routine work 5.Do not allowed alone work 6.Frequent breaks 7.Proper supervision available 8.Correct PPE must use 9.Provide heat stress training 10.Conduct tool box talk 11.Proper signage for drinking water | 1 | 4 | 4 | Low | Site Eng. Supervisors, Foreman |
| 10 | Working at night time | Poor visibility due to darkness / poor illumination Lone worker | Operatives/staff | 3 | 4 | 12 | Medium | 1) Ensure proper night work permit is obtained 2) Proper lighting system should be in place 3) Ensure proper Communication / Coordination and close supervision 4) Industrial safe torches to be available for emergency situations 5) High visibility Traffic Vest for personnel’s in night shift 6) Avoid lone working and always ensure buddy system while working at night 7) Employees to be trained and understand McLaren Crises plan. | 1 | 4 | 4 | Low | Site Engr Supervisor Foreman |
| 11 | Use of Hand tools | Improper selection of hand tools. Damaged and Defective Hand tools Hand injuries Pinch point Ergonomics Repetitive strain injury | Operatives/Staff | 3 | 3 | 9 | Medium | - Hand tools should be visually inspected for defects, prior to use. - Never use damaged, blunt or broken tools to avoid injury. - Select right tools for right Job - Ensure no Homemade or makeshift tools to be used at site - Remove from service any tool that shows signs of damage or defect - Ensure Hand tools are Stored in accordance with the manufacturer’s instructions. - Ensure hands are not in direct line of fire while working with hand tools - Ensure appropriate PPE at all times. | 1 | 3 | 3 | Low | Site engg Supervisor Foreman |

Approved By:  Name                                           Position          __________ Signature ___________

RISK ASSESSMENT & CONTROL GUIDELINES

- Executing Steps:

  - Planning

- Construction and HSE team must ensure that hazard identification is complete.

- Construction and HSE team must prioritize the hazard issues, which are of significant in nature. (It means that risks have well established legal requirements, potentially high risks).

- Construction and HSE team to prepare the risk assessment plan for the priorities identified hazards for these potential high risks.

  - Risk Assessment

Risk is the probability of an event occurring in a given set of circumstances. The ‘event’ is an exposure to hazard. The hazard is the potential to cause harm. The risk assessment is the technique of evaluating not just the likelihood of an event occurring, but also the outcome will be in terms of injury, loss, damage or harm.

  - Risk Assessment Process

The process of carrying out a risk assessment should be as follows.

- Identify the hazards.

- Identify who might be harmed and how.

- Evaluate the risk and implement the control measures.

- Record the significant findings.

- Review the assessment and update if necessary.

  - Examination of the Hazards and Risk Associated

- Competent staff must be used in examining the risk associated with the identified hazard.

- Competent staff should examine following aspect to determine the risk involved:

- Examine the existing control measures in place.

- Identify employees at risk.

- Likelihood of risk.

- Severity

- Risk level and their tolerability.

  - Evaluating the risk:

Once the necessary information has been obtained on the hazards encountered by work activities, next stage is to access the risks.

    - Risk Rating Score

Risk rating score is a combination of two factors.

- The severity of the risk that could injure persons or cause damage to plant.

- The likelihood of the risk that it could happen (Probability).

RISK RATING = LIKELIHOOD X SEVERITY

SEVERITY

NOTE: Each activity has to be assessed for the risk value for determining the level of Severity and likelihood are mentioned in the table below.

|  | No Injury (1) | Minor Injury (2) | Moderate Injury (3) | Major Injury (4) | Catastrophic (5) |
| --- | --- | --- | --- | --- | --- |
| Rarely (1) | 1 | 2 | 3 | 4 | 5 |
| Unlikely (2) | 2 | 4 | 6 | 8 | 10 |
| Possible (3) | 3 | 6 | 9 | 12 | 15 |
| Likely (4) | 4 | 8 | 12 | 16 | 20 |
| Almost Certain (5) | 5 | 10 | 15 | 20 | 25 |

LIKELIHOOD

RISK LEVEL:

| Low | Medium | High |
| --- | --- | --- |